# Field Development Plan – Content Requirements and Assessment

All fact sheets should be read in conjunction with the Offshore Petroleum and Greenhouse Gas Storage Act 2006 (the OPGGS Act), associated regulations, relevant guidelines and policies (available on NOPTA’s website).

This Fact Sheet provides information on the content requirements for Field Development Plans (FDPs) set out in the Offshore Petroleum and Greenhouse Gas Storage (Resource Management and Administration) Regulations 2025 (Cth) (the Regulations). All section references in this fact sheet pertain to the Regulations unless otherwise stated.

Field development planning is one phase of a typical lifecycle of a petroleum resource. An FDP is one of the Joint Authority (JA) approvals required to develop petroleum resources in offshore areas (for more information please see the Fact Sheet – Developing a Petroleum Resource).

The key objectives of an FDP are to document and communicate the subsurface development strategy and management plan, including wells and the associated sub-sea and surface facilities and activities.

The Titles Administrator, as technical advisor to the JA, assesses FDP applications and provides advice to the JA.

For more information about the field development planning process and the regulatory submission requirements for an FDP, refer to the FDP Regulatory Submission Requirements Fact Sheet

# What should an FDP Contain?

## Introduction

The appropriate content is critical to fulfilling the criteria for acceptance of an FDP under section 44 of the Regulations.

Subsection 45(1) of the OPGGS Act specifies the minimum mandatory content requirements of FDPs and describes the types of information required rather than the precise information and data to be included. This is consistent with the principles of objective-based regulation and provides flexibility for different development projects and concepts.

The key objectives of an FDP are to document and communicate the subsurface development strategy, including a description of project phasing, schedules and facilities requirements (i.e. wells and associated subsea and surface infrastructure to support production, injection, processing, storage and export operations).

The FDP must demonstrate that pool management in the field will be conducted in a manner consistent with good oilfield practice and optimum long-term recovery of the petroleum. For further information refer to the Assessment section below.

Undertaking early engagement activities with the Titles Administrator, including submission of a pFDP, will help ensure clarity of requirements, assessment processes, timeframes and expectations (see also the the FDP Regulatory Submission Requirements Fact Sheet).

## Content

FDPs are underpinned by extensive geoscientific and engineering datasets, reflecting complex, detailed, and extensive activities and studies. The FDP should contain sufficient detail of these activities and studies for the JA to make informed decisions.

Applicants should include in the FDP submission a summary of key licence, field and development data as per the table shown in Appendix A.

Appendix B provides an example of an FDP contents page, which may be used as a guide to address the regulatory requirements and support the Titles Administrator’s assessment of the FDP. The following is a general summary of the expected content:

An executive summary, with an overview of the field and development pathways.

Background information on the licence location, regional context, and details of current and potential future developments.

A reservoir description, which describes all aspects of the subsurface environment including the characteristics of associated fields and petroleum pools.

A description of reservoir development studies and plans for resource recovery, volumetric estimates, and production and injection forecasts.

The development concept screening and demonstration that the selected development concept supports optimum long-term recovery of petroleum from the petroleum pools.

A project and facilities overview, including anticipated schedule.

Final cost estimates and expenditure phasing for a commercial development and annual cash flow forecast.

An operational overview, including field and resource recovery management (including project governance arrangements for keeping records).

## Supporting data

Field development planning is usually founded on extensive data sets. These data sets must be examined and tested to ensure that they are representative of the range of possible technical, operational and economic outcomes. Demonstrated confidence in the quality of the data sets and understanding of the associated uncertainties is fundamental to the success of the project (refer below). The acquisition and validity of data should be discussed.

The FDP should discuss the most important components of the data, set in a quality-assured, processed and interpreted format. The presentation of raw datasets is usually not necessary.

Titleholders should submit resource estimates, production forecasts and economic data using the templates provided in Appendices C and D. Copies of these spreadsheets can be found on the Titles Administrator’s website at https://www.nopta.gov.au/forms-and-templates/reporting-templates.html.

## Uncertainty management and reporting

Uncertainties relevant to the efficient recovery of petroleum and the economic success of the project, including those in the broader geographic area, should be clearly presented.

The description of key uncertainties should be accompanied by information on how they will be managed. For example, explaining how specific outcomes can be mitigated, managed or minimised by the proposed development concept.

Each titleholder will have their own process for assessing, managing and reporting uncertainty. The table below is a suggested minimum of what should be included:

| Uncertainty type | Minimum representation |
| --- | --- |
| Initially in-place volumes | Low/P90, Mid/P50, and High/P10 by product |
| Recoverable volumes | 1P, 2P and 3P reserves by product |
| Contingent resources | 1C, 2C and 3C estimates by product |
| CAPEX/OPEX | Low, Mid and High estimates |
| Structural mapping of top reservoir | Low, Mid and High realisations |
| Fluid contacts | Low, Mid and High realisations |
| Reservoir stratigraphy and geology | Low, Mid and High realisations |
| Initial field plateau production rates | Low, Mid and High realisations |

# How are FDPs Assessed?

## Introduction

Section 44 of the OPGGS Act sets out the criteria for the acceptance of an FDP by the JA. It requires that the plan includes the matters listed in section 45 of the OPGGS Act and that it demonstrates that the pool will be managed in a manner that is consistent with good oilfield practice and compatible with optimum long-term recovery of petroleum.

## Optimum recovery and good oilfield practice

Key to the acceptance of an FDP is the demonstration of ‘optimum long-term recovery’ and ‘good oilfield practice’ – subsection 44(b).

Optimum long-term recovery of petroleum is delivered by a field development plan designed to achieve an acceptable balance between maximising ultimate resource recovery, project costs and economics, and associated risks. The development should be sufficiently flexible to mitigate key uncertainties and should consider both the standalone and area impacts and opportunities.

Good oilfield practice is defined in section 7 of the OPGGS Act as ‘all those things that are generally accepted as good and safe in: (a) the carrying on of exploration for petroleum; or (b) petroleum recovery operations.  This is generally understood to include all industry practices and technology generally accepted as safe, economical and efficient in exploring for and exploiting petroleum. The underlying concepts of resource conservation, energy efficiency and maximising economic ultimate recovery are included in the concept, but it is not limited to these.

## Technical aspects

The technical aspects considered by the Titles Administrator when assessing an FDP and formulating its advice to the JA include:

The application of improved/enhanced resource recovery processes and techniques

The conservation of reservoir energy and minimisation of resource ‘waste’ e.g. via discharges, flaring, venting or fuel usage

The application of new technology to improve recovery and enable efficient production

The area impacts of the development and provision of third-party infrastructure access, including neighbouring licences and permits

The plans for reservoir surveillance, production testing, field performance monitoring and routine analysis

The maturation plans for Contingent Resources, near-field tiebacks and future exploration potential.

## Economic aspects

The economic aspects considered by the Titles Administrator when assessing an FDP and formulating its advice to the JA, include:

The timing of first production

The full life-cycle cost of the development

The indicative decommissioning costs of the development, noting this is not approval of the final decommissioning strategy

The tariffs paid, or charged by, the development

The project life-cycle rate of return and net present value based on a mid-case production outcome and P50 cost estimate;

The sensitivities of the above to a range of outcomes, including breakeven commodity prices

The impact on any existing production or potential resource development projects

The economic cut-off and end-of-field-life rationale.

## Other aspects

Other aspects considered when assessing an FDP include:

The area impacts, including resource development benefits of the project

The development activity phasing and potential benefits

The late and end of field life technical and economic optimisation strategies

The utilisation of existing infrastructure and area impacts, including collaboration in general.

# More Information

If you have any specific questions, please contact NOPTA via titles@nopta.gov.au.

Please note: this document is intended as a guide only and should not be relied on as legal advice or regarded as a substitute for legal advice in individual cases.

# Version history

| Version | Date | Comment |
| --- | --- | --- |
| 1.0 | Mar 2026 | First Version Created |

Appendix A – Key Field Information Template.

| Category | Item | Key Information |
| --- | --- | --- |
| Title | Petroleum title |  |
|  | Titleholders and title interests |  |
|  | Geographical location |  |
|  | Water depth (m) |  |
|  | Existing infrastructure |  |
| History | Discovery date |  |
|  | Concept Select date |  |
| Reservoir | Principal producing formation(s) |  |
|  | Geological depositional setting |  |
|  | Datum depth |  |
|  | Depth of top structure |  |
|  | Initial pressure and temperature |  |
|  | Field area (km2) |  |
|  | Fluid contacts (e.g. OWC, GOC) |  |
|  | Average porosity, permeability and NTG |  |
|  | PVT properties (e.g. API, CGR, GOR, etc) |  |
|  | Recovery Factor(s) |  |
| Facilities | Development well count and types |  |
|  | Subsea infrastructure facilities |  |
|  | Processing, treatment facilities and capacities |  |
|  | Export and storage facilities and capacities |  |
|  | Future development and drilling phases |  |
|  | Third-party access provisions |  |
|  | Flaring, venting and emissions controls |  |
| Project | Project scope – key elements |  |
|  | Final Investment Decision date (forecast) |  |
|  | First production date (forecast) |  |
|  | Forecast end of field life (forecast range) |  |
|  | Key project schedule dates/durations |  |
|  | Forecast production profile (high, mid, low) |  |
|  | Overall capital costs and breakdown |  |
|  | Forecast annual operating costs |  |
|  | Forecast decommissioning costs |  |
|  | Development spending phases |  |

Appendix B – Detailed FDP and preliminary-FDP content list.

Appendix C.  Petroleum Resource Reporting Template

Appendix D.  Production / Injection Forecast Template

See FDP Regulatory Submission Requirements Fact Sheet.

Ibid

See FDP Regulatory Submission Requirements Fact Sheet

Ibid